Abstract

This project presents the Automated Equipment Health Monitoring System (AEHMS), an end-to-end predictive maintenance solution designed for defense and industrial vehicles and equipment. The system addresses Problem Statement 49 from the Indian Army Compendium of Future Core Technologies, which calls for transitioning from corrective to preventive maintenance philosophy. AEHMS leverages modern IoT sensors including DHT11 temperature/humidity sensors, SW-420 vibration sensors, pressure sensors, and level sensors integrated with Raspberry Pi 4 edge computing. Real-time data pipelines utilize MQTT protocol via HiveMQ cloud broker for secure communication, FastAPI backend for processing, and InfluxDB time-series database for efficient storage and analytics.

The Random Forest classifier model achieved 82.5% accuracy with 81.3% precision and 79.8% recall on test data. Five-fold cross-validation confirmed consistent performance with mean accuracy of 81.8% ± 2.1%. In comparative trials, AEHMS reduced missed failures by 30% (p < 0.05) and halved maintenance response time compared to legacy manual methods. The system provides 48-72 hour advance failure warnings with 95% confidence intervals. Implementation utilizes Streamlit for interactive dashboard visualization, providing real-time equipment status monitoring, maintenance alert notifications, and automated report generation in PDF and CSV formats. The modular hardware design costs ₹3,200 ($38) per unit, demonstrating significant cost advantages over commercial solutions while achieving superior accuracy in the sub-$100 category. The system successfully fulfills all requirements outlined in the Compendium including digital logbooks, automated preventive maintenance schedules, inventory management, and ERP integration capabilities.

TABLE OF CONTENTS

| CHAPTER NO | TITLE | PAGE NO |
| --- | --- | --- |
|  | ABSTRACT | v |
|  | TABLE OF CONTENTS | vi |
|  | LIST OF FIGURES | viii |
|  | LIST OF TABLES | ix |
| 1. | INTRODUCTION 1.1 BACKGROUND AND MOTIVATION 1.2 SCOPE AND LIMITATIONS 1.3 PROBLEM STATEMENT 1.4 MOTIVATION 1.5 METHODOLOGY OVERVIEW 1.6 ORGANIZATION OF THE THESIS 1.6.1Logical Flow of the Research Work 1.6.2Contribution Mapping Across Chapters 1.6.3Chapter Overview Summary | 1 1 2 3 3 3 4 4 5 5 |
| 2. | LITERATURE REVIEW 2.1 INTRODUCTION TO RELATED WORK 2.2 SURVEY OF EXISTING SYSTEMS 2.3 RESEARCH GAPS IDENTIFIED 2.4 OBJECTIVES OF THE STUDY 2.5 SUMMARY | 6 6 6 11 11 12 |
| 3. | RESEARCH GAPS OF EXISTING METHODS |  |
| 4. | PROPOSED METHODOLOGY 3.1 METHODOLOGY OUTLINE 3.2 TOOLS, TECHNOLOGIES, PLATFORMS USED 3.3 PROPOSED ARCHITECTURE 3.4 DATA COLLECTION & PREPROCESSING 3.4.1 Data source and composition 3.4.2 Feature selection and structuring 3.4.3 Preprocessing workflow 3.4.4 Sequence construction for temporal learning 3.4.5 Data splitting and validation strategy 3.5 DATA SET DESCRIPTION 3.5.1 Data Acquisition and Understanding 3.5.2 Feature Engineering and Sequence Construction 3.5.3 Synthetic Data Generation Usi 3.6 WORKFLOW DIAGRAM 3.7 SUMMARY | 13 13 13 14 16 16 17 17 17 17 18 18 19 20 20 21 |
| 5. | OBJECTIVES |  |
| 6. | SYSTEM DESIGN AND IMPLEMENTATION 4.1 MODULE-WISE IMPLEMENTATION DETAILS 4.1.1 Data Preparation and Encoding Module 4.1.2 Sequence Construction and Windowing Module 4.1.3 Model Architecture and Model Training 4.2 ALGORITHMS 4.3 SCREENSHOTS 4.4 INTEGRATION OF COMPONENTS 4.5 SUMMARY | 22 22 22 23 24 24 26 29 30 |
| 7. | TIMELINE FOR EXECUTION OF PROJECT |  |
| 8. | OUTCOMES |  |
| 9. | RESULTS AND DISCUSSION 5.1 EXPERIMENTAL SETUP ANDENVIORNMENT 5.2 PERFORMANCE METRICS USED 5.3 COMPARATIVE RESULTS 5.4 ANALYSIS AND INTERPRETATION OFRESULTS 5.5 DISCUSSION ON OUTCOMES 5.6 SUMMARY | 31 31 32 33 34 35 37 |
|  | CONCLUSION AND FUTURE WORK SUMMARY OF CONTRIBUTIONS KEY FINDINGS LIMITATIONS OF THE STUDY SUGGESTIONS FOR FUTURE RESEARCH SUMMARY | 38 38 39 39 40 41 |
|  | REFERENCES | 42 |
|  | APPENDIX- A | 45 |
|  | APPENDIX- B | 48 |
|  | APPENDIX- C | 55 |
|  | APPENDIX- D | 56 |

LIST OF FIGURES

| Sl. No. | Figure Name | Caption | Page No. |
| --- | --- | --- | --- |
| 1 | Figure 3.1 |  | 15 |
| 2 | Figure 3.2 |  | 16 |
| 3 | Figure 3.3 |  | 18 |
| 4 | Figure 3.4 |  | 20 |
| 5 | Figure 4.1 |  | 22 |
| 6 | Figure 4.2 |  | 23 |
| 7 | Figure 4.3 |  | 26 |
| 8 | Figure 4.4 |  | 26 |
| 9 | Figure 4.5 |  | 27 |
| 10 | Figure 4.6 |  | 27 |
| 11 | Figure 4.7 |  | 28 |
| 12 | Figure 4.8 |  | 28 |
| 13 | Figure 4.9 |  | 29 |
| 14 | Figure 5.1 |  | 32 |
| 15 | Figure 5.2 |  | 34 |
| 16 | Figure 5.3 |  | 35 |
| 17 | Figure 5.4 |  | 36 |
| 18 | Figure 5.5 |  | 37 |

LIST OF TABLES

| Sl. No. | Table Name | Table Caption | Page No. |
| --- | --- | --- | --- |
| 1 | Table 5.1 |  |  |

Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| AEHMS | Automated Equipment Health Monitoring System |
| AFV | Armoured Fighting Vehicle |
| API | Application Programming Interface |
| ATGM | Anti Tank Guided Missile |
| AUC-ROC | Area Under Curve - Receiver Operating Characteristic |
| BMCS | Bi-Modular Charge System |
| COTS | Commercial Off-The-Shelf |
| CSV | Comma-Separated Values |
| DHT | Digital Humidity and Temperature |
| DRDO | Defence Research and Development Organisation |
| EME | Electronics and Mechanical Engineers |
| ERP | Enterprise Resource Planning |
| FPA | Focal Plane Array |
| GPIO | General Purpose Input/Output |
| GPS | Global Positioning System |
| HAA | High Altitude Area |
| HTML | HyperText Markup Language |
| IFF | Identification of Friend or Foe |
| InfluxDB | Time-Series Database |
| IoT | Internet of Things |
| IRNSS | Indian Regional Navigation Satellite System |
| JSON | JavaScript Object Notation |
| LAC | Line of Actual Control |
| LTE | Long Term Evolution |
| MAWS | Missile Approach Warning System |
| MCCS | Mobile Cellular Communication System |
| MGB | Main Gear Box |
| ML | Machine Learning |
| MQTT | Message Queuing Telemetry Transport |
| MTBF | Mean Time Between Failures |
| NVD | Night Vision Device |
| OEM | Original Equipment Manufacturer |
| PESTEL | Political, Economic, Social, Technological, Environmental, Legal |
| QoS | Quality of Service |
| RBAC | Role-Based Access Control |
| REST | Representational State Transfer |
| RF | Random Forest |
| RH | Relative Humidity |
| ROI | Return on Investment |
| SAM | Surface to Air Missile |
| SAR | Synthetic Aperture Radar |
| SDG | Sustainable Development Goal |
| SDK | Software Development Kit |
| SQLite | Structured Query Language Lite |
| SSL | Secure Sockets Layer |
| SVM | Support Vector Machine |
| TBA | Tactical Battle Area |
| TI | Thermal Imaging |
| TLS | Transport Layer Security |
| UAV | Unmanned Aerial Vehicle |
| URL | Uniform Resource Locator |
| VPN | Virtual Private Network |
| XML | Extensible Markup Language |